Week 2 Ethernet Standards Assignment

Dominic Bumpus
Network Management
Brandon Kemp
11/01/2025

# Ethernet Standards

Ethernet technology serves as the foundation for most wired networking systems used today. Over the years, various Ethernet standards have emerged to allow for increasing data transfer rates, different cabling types, and extended transmission distances. The standard for these cabling types includes four important standards: 10BASE-T, 100BASE-TX, 1000BASE-T, and 10GBASE-LR, which have different cable types, speeds, distances, and significance. (GeeksforGeeks, 2025)

	The 10BASE-T standard marked a major shift from coaxial to twisted-pair cabling, utilizing Category 3 or better cables to deliver speeds of up to 10 megabits per second over a maximum distance of 100 meters. This transition made Ethernet installation simpler and more affordable for offices, establishing the foundation for future Ethernet standards that continued to use twisted-pair cabling (Law, 2010).

	Building upon this, 100BASE-TX introduced “Fast Ethernet”, which increased data transmission rates to 100 Mb/s while maintaining the same 100-meter cable length using Category 5 twisted-pair cabling. This standard gained popularity in the 1990s because it enabled faster data exchange without requiring new wiring infrastructure, promoting widespread adoption in business and educational networks (Upravnik, 2022).

	The 1000BASE-T standard, known as “Gigabit Ethernet,” further increased the speed to 1 gigabit per second using Category 5e or higher twisted-pair cables. Like its predecessors, it supported cable runs up to 100 meters. This standard is notable for its ability to deliver high performance over existing copper wiring, striking a balance between speed and cost efficiency (Mailali, 2025).

	Finally, 10GBASE-LR extended Ethernet into high-speed optical networking. Operating at 10 Gigabits, it uses single-mode optical fiber to reach distances of up to 10 kilometers. This development allowed Ethernet to be used in large-scale enterprise, data center, and metropolitan area networks, where both speed and range are critical (Law, 2010).

	Overall, each of these four standards, 10BASE-T, 100BASE-TX, 1000BASE-T, and 10GBASE-LR, represents an important milestone in Ethernet’s progression. From the introduction of twisted-pair cabling to the advent of high-speed optical communication, the IEEE 802.3 standards have continually evolved to meet the growing demands of modern networking environments.

# References

GeeksforGeeks. (2025, October 3). What is Ethernet? GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/what-is-ethernet/\

Law, D. (2010). *IEEE 802.3 Ethernet Technologies* (Version 1.0). IEEE 802.3 Working Group.

Mailali. (2025, September 10). IEEE Ethernet Standards Guide | Networking Basics explained - Cables and Kits - Learning Center. Cables and Kits - Learning Center. https://www.cablesandkits.com/learning-center/making-sense-of-ieee-ethernet-standards/?srsltid=AfmBOor4S9U-U9ON6cAHSYnk7y9K_1g4x6uBXfwO_7W25raIPuUUG7EQ
Upravnik. (2022, December 18). IEEE Ethernet standards. Study CCNA. https://study-ccna.com/ieee-ethernet-standards/