Dominic Bumpus IP Addressing and Subnetting Quiz

Dominic Bumpus

Trine University

IS---5203-202
Brandon Kemp

November 7, 2025

The IP Address has been given as 192.168.165.17/29. As the IP has /29 on the end this means that 29 of the 32 bits are 1s and the remaining 3 are 0s which gives us the subnet mask 255.255.255.248. The subnet size is equal to 256 -248 which means there are 8 addresses per subnet. The last octet multiples will be 0, 8, 16, 24, and as last octet of the IP is 17 and the last octet of the mask is 248 which means the network IDs last octet will be 16. This means the network ID will be 192.168.165.16/29. The Broadcast ID is calculated by taking the Network ID + (Block Size-1) which means it will be 192.168.165.16 + 7 which means the Broadcast ID is 192.168.165.23. The IP is in the subnet 192.168.165.16/29 which means the IP will have a usable range of 192.168.165.17 through 192.168.165.22.

References:

GeeksforGeeks. (2025, July 23). Subnet mask cheat sheet. GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/subnet-cheat-sheet/

GeeksforGeeks. (2025, July 11). Finding Network ID of a Subnet (using Subnet Mask). GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/finding-network-id-of-a-subnet-using-subnet-mask/

GeeksforGeeks. (2025, July 23). How to calculate number of host in a subnet? GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/how-to-calculate-number-of-host-in-a-subnet/

GeeksforGeeks. (2025, July 23). IP Custom Subnet Masks. GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/ip-custom-subnet-masks/