Dominic Bumpus VLAN Segmentation

Dominic Bumpus

Trine University

IS---5203-202
Brandon Kemp

November 7, 2025

IEEE 802.1Q VLAN Tagging is one of the most important technologies in today’s networking methodologies as it makes it possible to extend a VLAN across multiple switches, maintain separate broadcast domains, and transport traffic from VLANs over a singular physical link. VLANs are a logical partition of a layer 2 network that can span multiple physical locations and isolate traffic into separate broadcast domains. IEEE 802.1Q Tagging is what allows these VLANs to function across trunk links. Trunks carry traffic from several VLANs at the same time, but without tagging these switches would not know which VLAN an incoming transmission belongs to when it arrives at a trunk link. This is solved with VLAN tags which contain the VLAN ID the transmission belongs to, so the trunk link knows where to send the data (GeeksforGeeks, 2025).

VLAN tagging also allows for network segmentation and performance improvements. Each VLAN is its own broadcast domain which prevents unnecessary traffic from overwhelming the network it’s transmitting data on. These domains are scalable because they can span multiple switches, traffic remains isolated, and a network can support thousands of VLANs. In modern networks, segmentation is used for security, performance improvements, compliance with regulations, and overall VLAN tagging is a critical technology for this all to happen (GeeksforGeeks, 2025).

Moreover, VLAN tagging is required for Inter-VLAN routing, supports technologies like voice VLANs, and enables infrastructure services like management and native VLAN handling. Inter-VLAN routing requires 802.1q tagging because a router sub-interface must understand incoming tagged frames to route between VLANs and send tagged frames back to the proper switch for the correct VLAN (GeeksforGeeks, 2023). VLAN tagging also supports voice VLANs which separate voice and data traffic on the same port using tagging. This is essential because IP phones need low latency and high priority, a PC can share a single port with a VOIP phone, and the phone tags voice traffic while data remains on another VLAN. Tagging also enables various infrastructure such as native VLANs, management VLANs for SSH, SNMP, and Syslog, as well as the separation of sensitive traffic such as security camera footage (GeeksforGeeks, 2025).

References:

Cisco. (n.d.). Chapter 6: VLANs. In Cisco. Retrieved November 16, 2025, from http://vapenik.s.cnl.sk/pcsiete/CCNA2/06_VLANs.pdf

GeeksforGeeks. (2025, October 3). Virtual LAN (VLAN). GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/virtual-lan-vlan/

GeeksforGeeks. (2023, May 4). Access and trunk ports. GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/access-trunk-ports/

GeeksforGeeks. (2025, July 15). Types of virtual LAN (VLAN). GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/types-of-virtual-lan-vlan/

GeeksforGeeks. (2023, May 2). Inter VLAN routing by Layer 3 switch. GeeksforGeeks. https://www.geeksforgeeks.org/computer-networks/inter-vlan-routing-layer-3-switch/